AAFC-Engineering Review of Posting – ERO 025-0219 ref# 1000334473 - 780 Baseline Road inc. Request for Permit to Take on Water – Environmental Registry of Ontario

April 11, 2025

Table 4-1 – Dewatering Discharge Management Plan

- Given the fluctuation of groundwater levels throughout the year, further clarification is needed regarding the definition of baseline parameters that will be used for monitoring groundwater levels in existing or proposed monitoring wells. It is anticipated that a similar approach to that listed in Table 3-1 will be adopted, consisting of a series of readings taken over several months to capture seasonal fluctuations in groundwater levels.

- If trigger levels are exceeded, this could result in irreversible damage to crops, which AAFC cannot tolerate as an acceptable level of risk. We recommend coordinating with AAFC staff and potentially incorporating a safety factor into the trigger levels to allow for a proactive response in implementing the mitigation measures outlined in the table.

5.1 – Central Experimental Farm Lands

- Considering that the dewatering rate will be significantly higher in the short term (with a peak of 1.15 million LPD), please clarify the statement regarding the unlikelihood of potential impacts on the groundwater table at CEF during short-term construction activities.

- To aid in understanding the short-, medium-, and long-term impacts, the report should include cross-sections illustrating the drawdown of groundwater in the Zone of Influence (ZOI) under both temporary and permanent conditions. This will provide a graphical representation of groundwater level reductions across the fields of the CEF.

6.0 – Conclusions and Recommendations

- It is stated that there is a long-term potential impact on groundwater levels at CEF, and AAFC should be an active stakeholder in the design process of the foundation system for the new development.

We also recommend that AAFC remain an active stakeholder in the design process of the foundation system (and other relevant building components) of the proposed development.